Device Driver to Detect W3150A or W3150A+

At main function of the firmware of MCU that you are using

int main(void)

{

SOCKET i;

system_init();			//Initialization of MCU

iinchip_init();

check_3150model();		//check WIZnet Model (W3150A / W3150A+)

iinchip_init();

;

init_timer();

ㅡ

ㅡ

.

.

.

.

}

If you add below part to the main() function, it is possible to detect W3150A or W3150A+ automatically.

void check_3150model(void)

{

uint8 a, tmp;

tmp = IINCHIP_READ(COMMON_BASE+0x041E);

IINCHIP_WRITE(COMMON_BASE+0x041E, 0x5a);

a = IINCHIP_READ(COMMON_BASE+0x041E);

IINCHIP_WRITE(COMMON_BASE+0x041E, tmp);

PRINT("WIZnet Model : ");

if (a == 0x5a)

{

model_type = _F_3150A_PLUS_;

PRINTLN("W3150A+");

}

else

{

model_type = _F_3150A_;

PRINTLN("W3150A");

}

//else evb_soft_reset();

}

In the device driver source code that we are providing, the distinction of W3150A and W3150A+ can be processed by ‘if’ syntax.

Ex) in the ‘socket.c’ file of device driver

uint8 socket(SOCKET s, uint8 protocol, uint16 port, uint8 flag)

{

uint8 ret;

#ifdef __DEF_IINCHIP_DBG__

printf("socket()\r\n");

#endif

if ((protocol == SOCK_STREAM) || (protocol == SOCK_DGRAM) || (protocol == SOCK_ICMPM) || (protocol == SOCK_IPL_RAWM) || (protocol == SOCK_MACL_RAWM) || (protocol == SOCK_PPPOEM))

{

if(model_type == _F_3150A_)

{

if (IINCHIP_READ(SOCK_STATUS(s)) != SOCK_CLOSED) close(s);

}

.

.

.